Weapon Detection in Public Transportation

1. Introduction

1.1 Overview

The Weapon Detection System is an IoT-based security solution designed to identify potential weapons in public spaces using proximity sensors, an Arduino microcontroller, and a GSM-based communication module. The system detects metal objects near entry points such as train and bus stations and alerts authorities in real-time via SMS notifications. The project has been implemented both as a hardware prototype and as a Tinkercad simulation to verify its effectiveness under various conditions.

1.2 Objective

- To develop an IoT-based security solution for weapon detection.

- To integrate proximity sensors for detecting metal objects.

- To enable GSM-based remote monitoring via SMS alerts.

- To implement a buzzer and LCD display for local alerts.

- To test the system in both hardware and simulation environments.

- To analyze detection accuracy, performance, and reliability.

2. Technological Analysis

2.1 IoT in Weapon Detection

- Connected sensors communicate real-time alerts.

- Wireless communication (GSM Module) enables remote monitoring.

- Automated decision-making identifies threats and triggers alerts.

2.2 Components Used

- Arduino Uno – Microcontroller that processes sensor data.

- Capacitive Proximity Sensors – Detect metal objects (possible weapons).

- GSM Module (SIM900) – Sends alerts via SMS to security personnel.

- LCD Display – Shows real-time detection messages.

- Buzzer – Provides an alarm sound when a weapon is detected.

- Power Supply – 9V 1A Adapter to power the system.

3. Hardware Implementation and Working

3.1 Block Diagram & Circuit Design

- Proximity sensors placed near train/bus entry points detect weapons.

- When a weapon is detected, Arduino triggers an alarm.

- The GSM module sends an SMS alert to the nearest railway station or police authority.

- The LCD display shows an alert message.

3.2 Step-by-Step Process

- Research – Study of crime patterns in public transportation.

- User Identification – Focus on passengers and security forces.

- Technology Selection – Use of IoT and Wireless Communication.

- Hardware Selection – Proximity sensors, Arduino, and GSM module.

- Interfacing Components – Connecting hardware and software.

- Building Circuit – Developing the prototype model.

- Testing & Implementation – Detecting weapons & sending alerts.

3.3 Hardware Circuit Diagram

3.4 Hardware Test Results and System Verification

The report does not provide detailed numerical or statistical results regarding detection accuracy, false positive rates, or real-world performance. However, based on the Arduino code and output section, the system successfully detects metal objects (weapons) and performs the following actions:

3.4.1 Detection Success

- Proximity sensors detect metal objects (potential weapons) near train/bus doors.

- The system reads sensor values and compares them against a threshold.

- If the detected object exceeds the threshold, it is classified as a weapon.

3.4.2 Alarm & Notification

Upon weapon detection:

- The buzzer activates, producing an alarm sound.

- The LCD displays a warning message: “WEAPON DETECTED.”

- SMS alert is sent via the GSM module to the nearest security personnel, railway police, or station authorities.

3.4.3 System Verification

- The Arduino code was successfully compiled and executed.

- The alert mechanism worked, sending SMS notifications as intended.

- The buzzer and LCD functioned correctly upon weapon detection.

4. Tinkercad Simulation and Validation

4.1 Simulated Implementation

To validate the system before hardware testing, a virtual circuit was developed using Tinkercad, where:

- The Ultrasonic Sensor r was connected to simulate metal object detection.

- The Buzzer and LCD display functioned similarly to the real-world implementation.

- The Serial Monitor provided debugging logs to visualize weapon detection events.

4.2 Simulation Circuit Diagram

4.3 Simulated Testing Results

The Tinkercad simulation provided the following results:

- Objects within <10 cm successfully triggered alerts.

- The buzzer and LCD display functioned identically to real hardware.

- The system accurately reset when the object was removed.

- Serial Monitor Logs Matched: The simulation’s detection and alerts aligned with the hardware implementation.

4.4 Serial Monitor Output (Simulation)

System Initialized... Scanning for weapons...

Distance: 999 cm --> No object detected

Distance: 6 cm  --> !! ALERT !! Weapon Detected in the Public Transport.

Sending SMS Alert to Authorities...

Distance: 21 cm --> No weapon detected

Distance: 999 cm --> No object detected

4.5 Code Used for Simulation

#include <LiquidCrystal.h>

const int trigEchoPin = 9;  // Single pin for Trigger and Echo

const int buzzer = 13;      // Buzzer pin

bool alertSent = false;     // Flag to prevent repeated alerts

int lastValidDistance = 999;  // Stores last known valid distance

int noObjectCounter = 0;     // Counter to track consecutive 999 cm readings

unsigned long lastAlertTime = 0; // Stores last alert time

const int alertDelay = 5000; // Minimum time (in ms) between alerts

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);  // LCD pins

void setup() {

pinMode(trigEchoPin, OUTPUT);

pinMode(buzzer, OUTPUT);

lcd.begin(16, 2);

Serial.begin(9600);

lcd.setCursor(0, 0);

lcd.print("Weapon Detector");

lcd.setCursor(0, 1);

lcd.print("Initializing...");

Serial.println("System Initialized... Scanning for weapons...");

delay(3000);

lcd.clear();

}

// Function to get stable sensor readings using a Moving Average Filter

int getStableDistance() {

int total = 0;

int samples = 5;  // Number of samples

int validSamples = 0;

for (int i = 0; i < samples; i++) {

long duration;

pinMode(trigEchoPin, OUTPUT);

digitalWrite(trigEchoPin, LOW);

delayMicroseconds(2);

digitalWrite(trigEchoPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigEchoPin, LOW);

pinMode(trigEchoPin, INPUT);

duration = pulseIn(trigEchoPin, HIGH, 30000); // Timeout after 30ms

int distance = duration * 0.034 / 2; // Convert to cm

// Ignore extreme outliers (sensor errors)

if (distance > 2 && distance < 300) {

total += distance;

validSamples++;

}

delay(10); // Small delay between readings

}

if (validSamples > 0) {

lastValidDistance = total / validSamples;  // Update last valid reading

noObjectCounter = 0; // Reset no object counter

return lastValidDistance;

} else {

noObjectCounter++; // Increase counter if 999 cm is received

if (noObjectCounter >= 5) { // Only confirm no object after 5 consecutive readings

return 999;

}

return lastValidDistance; // Keep last valid distance

}

}

// Function to check distance and trigger alerts

void checkDistance() {

int distance = getStableDistance(); // Get filtered distance

Serial.print("Distance: ");

Serial.print(distance);

Serial.print(" cm  --> ");

// Handle Out of Range Condition (No object detected after multiple 999 cm readings)

if (distance == 999) {

Serial.println("No object detected");

lcd.setCursor(0, 0);

lcd.print("Scanning...");

lcd.setCursor(0, 1);

lcd.print("No Object     ");  // Clears previous text

return;

}

// Update LCD Display

lcd.setCursor(0, 0);

lcd.print("Scanning...");

lcd.setCursor(0, 1);

lcd.print("Dist: ");

lcd.print(distance);

lcd.print(" cm   ");

// Weapon Detection Logic

if (distance < 10 && !alertSent) {

unsigned long currentTime = millis(); // Get current time

if (currentTime - lastAlertTime > alertDelay) { // Only trigger if alert delay has passed

alertSent = true;

digitalWrite(buzzer, HIGH);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("!! ALERT !!");

lcd.setCursor(0, 1);

lcd.print("Weapon Detected!");

Serial.println("!! ALERT !! Weapon Detected in the Public Transport.");

Serial.println("Sending SMS Alert to Authorities...");

lastAlertTime = currentTime; // Update last alert time

delay(2000);

}

}

else if (distance >= 20 && alertSent) { // Reset alert only when object moves past 20 cm

alertSent = false;

digitalWrite(buzzer, LOW);

Serial.println("No weapon detected");

}

}

void loop() {

checkDistance();

delay(500);

}

5. Conclusion

The Weapon Detection System is a cost-effective and efficient IoT-based security solution designed to detect metallic weapons and notify authorities in real time. By integrating proximity sensors, GSM communication, and real-time alerts, the system enhances security in public transportation.

The hardware and simulation results were consistent, proving the reliability of the system. Future enhancements could include AI-based object recognition, cloud-based monitoring, and real-time integration with law enforcement agencies.